QA/Test Report

Equipment name	:  Silar coating unit				Date:

Model No./ Serial no.	:

Customer/Job card No.	……………………………………………….

Standard Test Procedure:

(All the tests are performed based on the theory of the respective experiment and/or by calibrated instruments.)

| Sl No | Specific Test Conducted | Test Result | Specification/ Accepted values | Pass/Fail | Remarks |
| --- | --- | --- | --- | --- | --- |
| 1. | Basic Inspection |  | Model No., Painting , Finishing , stability and other visual parameters |  |  |
| 2. | Vibration and noise checking |  | Run a sample program and check |  |  |
| 3. | Keyboard/ Touch screen checking |  | Enter all the alpha numeric values and check its functioning |  |  |
| 4. | Up-Down travel and limit checking |  | Move the sample holder up side and down side till it reaches the limit |  |  |
| 5. | Travel length |  | Verify the travel length by running a sample program with specific travel length say 50 mm |  |  |
| 6. | Travel speed |  | Verify the travel speed by running a sample program with specific travel speed (both Dip and retrieval) in um/Sec and mm/Sec |  |  |
| 7. | Dip & dry duration |  | Run a sample program with specific Dip and Dry duration. Sec., Min. & Hour |  |  |
| 8. | Sample holder positioning. ( centering to the beaker) |  | Run a sample program with Dip in all the beakers and check the centering at each dip position. |  |  |
| 9. | Magnetic stirrer Function |  | Check the magnetic stirrer with beaker and run @ max rpm. |  |  |
| 10. | Heater Function |  | Turn on the heater and check the temperature. |  |  |
| 11 | PC –Software testing |  | Compose and run a sample program in PC-Software. |  |  |

Checked by						Approved by

………………….							…………..	…………